# User Manual-           Demand Flexibility analysis model

Developed by:                                                                                               MP Ensystems Advisory Pvt Ltd

June 2026

# Table of Contents

Table of Contents	2

1. Introduction	4

1.1 Purpose	4

1.2 Background: Demand Flexibility for DISCOMs	4

1.3 Key Features	4

2. System Requirements & Installation	6

2.1 Software Prerequisites	6

2.2 Installation Steps	6

3. Input Data Files	7

3.1 File Overview	7

3.2 Generation Stack File (Generation_stack.xlsx)	7

3.3 Demand Data File (demand.csv)	8

3.4 Market Rate File (market_rate.xlsx)	8

3.5 Renewable Energy File (RE.xlsx)	8

3.6 Input Parameters File (input_parameters.csv)	9

3.7 Generator Availability File (availability.xlsx)	9

4. Using the Application	10

4.1 Application Layout	10

4.2 Step-by-Step Workflow	10

Step 1: Upload Input Files	10

Step 2: Initialize Data	10

Step 3: Run the Analysis	11

Step 4: Review Results	11

5. Understanding the Optimization Model	12

5.1 Objective Function	12

5.2 Decision Variables	12

5.3 Constraints	12

5.4 Generation Cost Calculation	13

6. Output & Results	14

6.1 Generation Cost Chart	14

6.2 Summary Table	14

6.3 Generation Schedule Files	14

6.4 Base vs Optimised Demand Comparison	14

6.5 DF/DR Savings Heatmap	14

6.6 High-Cost Period Analysis	15

7. Application Page Summary	16

7.1 Main page	16

7.2 Flexibility Analysis	16

7.2.1 Generation Stack Area Chart	17

7.2.2 Demand Heatmap	17

7.2.3 Base vs Optimised Demand Comparison	18

7.2.4 DF/DR Savings Heatmap	18

7.3 IEX Market MCP Data Trend	18

7.3.1 DAM MCP Trend Chart	19

7.3.2 RTM MCP Trend Chart	19

7.4 Consumer Demand Analysis for High-Cost Periods	19

7.4.1 Consumer Analysis Table	20

7.4.2 Correlation: Consumer Load vs PU Generation Cost	21

7.4.3 Meter Data Heatmap	21

8. Troubleshooting	22

8.1 Common Errors and Solutions	22

8.2 Performance Tips	22

9. Updated File Structure	23

10. Glossary	24

11. Support & Maintenance	25

# 1. Introduction

The Demand Flexibility Analysis Model is a web-based analytical tool developed to help Indian Electricity Distribution Companies (DISCOMs) evaluate the economic and operational impact of demand-side management (DSM) strategies on power procurement and generation costs.

DISCOMs in India increasingly face the challenge of balancing grid stability with cost-efficiency, especially as renewable energy penetration rises and consumer demand patterns become less predictable. This model provides a structured, data-driven platform to simulate how flexing consumer demand — upward or downward — within defined bounds affects total generation cost.

## 1.1 Purpose

This tool enables DISCOM planners and energy managers to:

- Simulate how adjusting demand affects generation cost

- Quantify change in generation cost per unit change in demand (up or down), which can be used for estimating incentives

- Compare generation costs across multiple demand flexibility levels in a single run

- Identify high-cost time periods and pinpoint consumer meters that contribute to peak costs

- Evaluate the benefit of battery energy storage dispatch in reducing peak costs

## 1.2 Background: Demand Flexibility for DISCOMs

Demand Flexibility (DF) refers to the ability to shift, reduce, or increase electrical consumption in response to grid signals or economic incentives. For DISCOMs, this translates into:

- Demand Flexibility (DF): Shifting consumption across time slots within a day while keeping daily totals constant.

- Demand Response (DR): Temporary curtailment of load in response to high-cost or high-stress grid periods.

By modelling different flexibility levels (e.g., 5%, 10%, 15% of peak demand that can be shifted), DISCOMs can quantify savings from demand flexibility programmes before committing resources to field implementation.

## 1.3 Key Features

- Linear Programme (LP) based generation cost optimization using PuLP solver

- Multi-plant generation scheduling with ramping, technical minimum, and capacity constraints

- Battery Energy Storage System (BESS) dispatch modelling

- IEX market purchase integration

- High-cost period identification and consumer meter contribution analysis

- Interactive Streamlit dashboard with Plotly charts

# 2. System Requirements & Installation

## 2.1 Software Prerequisites

| Requirement | Details |
| --- | --- |
| Python | Version 3.9 or higher |
| Streamlit | >=1.35.0 (web app framework) |
| PuLP | >=2.7.0 (LP optimization solver) |
| Pandas / NumPy | >=2.0.0 / >=1.24.0 (data handling) |
| Plotly | >=5.18.0 (interactive charts) |
| openpyxl / xlrd | >=3.1.0 / >=2.0.0 (Excel file support) |
| Requests / lxml | >=2.31.0 (IEX data scraping) |
| SciPy | >=1.10.0 |

## 2.2 Installation Steps

- Ensure Python 3.9+ is installed on your system.

- Download or clone the model files to a local directory.

- Open a terminal/command prompt and navigate to the model directory.

- Install all dependencies by running:

pip install -r requirements.txt

- Launch the application by running:

streamlit run app.py

- The application will open automatically in your default web browser. If not, navigate to http://localhost:8501

Online Access (No Installation Required)

- If you prefer to use the tool without a local installation, the model is also hosted online and accessible directly at:

https://demandflexibilityapp.streamlit.app/

- Simply open the link in any modern web browser — no Python, no setup required. Note that the online version shares the same interface and functionality as the local installation, but large optimization runs may take longer depending on server load. Sensitive or proprietary input data should be handled with discretion when using the hosted version.

# 3. Input Data Files

The model requires six input files to be uploaded through the sidebar. All files must be prepared before launching an analysis run. Below is a description of each file, its format, and the required data structure.

## 3.1 File Overview

| File Name | Format | Description |
| --- | --- | --- |
| Generation_stack.xlsx | Excel (.xlsx) | Generator portfolio: capacity, costs, ramp rates, technical minimums |
| demand.csv | CSV | Hourly or sub-hourly projected demand (MW) for the analysis period |
| market_rate.xlsx | Excel (.xlsx) | IEX market purchase rates (₹/MWh) for each time slot |
| RE.xlsx | Excel (.xlsx) | Renewable energy generation profiles — Solar and Wind (MW) |
| input_parameters.csv | CSV | Model configuration parameters (flexibility levels, battery specs, costs, etc.) |
| availability.xlsx | Excel (.xlsx) | Generator availability flags (1 = available, 0 = unavailable) per time slot |

## 3.2 Generation Stack File (Generation_stack.xlsx)

This file defines the conventional thermal, gas, and hydro generation portfolio. Each row represents one generating unit.

| Column Name | Unit | Description |
| --- | --- | --- |
| Capacity | MW | Installed/contractual capacity of the generating unit |
| Technical_min | MW | Minimum generation level the unit must operate at |
| Fixed cost | ₹/hr | Fixed operating cost per hour (currently noted but not in objective function) |
| Variable cost | ₹/MWh | Variable generation cost — used in the LP objective function |
| Ramping_up | MW/slot | Maximum rate of upward change in generation between consecutive time slots |
| Ramping down | MW/slot | Maximum rate of downward change in generation between consecutive time slots (negative value) |

Note: Ramping limits are defined per time slot. For hourly data (daily_slots = 24), a ramp of 10 MW/slot means 10 MW/hour. For 15-minute intervals (daily_slots = 96), adjust accordingly.

## 3.3 Demand Data File (demand.csv)

This file contains the projected total consumer demand in MW for every time slot in the analysis period. It should be a single-column CSV file with one row per time slot.

| Parameter | Specification |
| --- | --- |
| Column | demand (MW) |
| Rows | Equal to num_slots (e.g., 8760 for hourly annual, 35040 for 15-min annual) |
| Data type | Numeric, positive values only |
| No header row required? | A header row is acceptable; the model reads by position |

## 3.4 Market Rate File (market_rate.xlsx)

This file contains the IEX market purchase rate for each time slot, representing the price at which the DISCOM can buy power from the exchange (Day-Ahead Market or Real-Time Market).

| Parameter | Specification |
| --- | --- |
| Column name | rate |
| Unit | ₹/MWh |
| Rows | Must match num_slots exactly |
| Typical source | IEX DAM/RTM data (can be fetched via built-in IEX scraper) |

Tip: If market rates are not available for a future period, use historical averages or forecasts. Market purchase is constrained by market_limit in input_parameters.csv.

## 3.5 Renewable Energy File (RE.xlsx)

This file contains generation profiles for renewable energy sources for each time slot. Both columns are required.

| Column Name | Unit | Description |
| --- | --- | --- |
| Solar | MW | Solar PV generation profile for each time slot |
| Wind | MW | Wind generation profile for each time slot |

RE generation is subtracted from gross demand to compute net demand for conventional plants. The model currently supports single aggregated solar and wind profiles. If RE forecasting is enabled in input_parameters.csv, the model will apply forecast-adjusted values.

## 3.6 Input Parameters File (input_parameters.csv)

This is the master configuration file. It contains all scalar and list parameters that control model behaviour. The file must have two columns: Parameter and Value.

| Parameter Name | Example Value | Description |
| --- | --- | --- |
| flexibility | [0, 0.05, 0.10, 0.15] | List of flexibility levels to simulate. 0 = base case (no flexibility), 0.05 = ±5% demand shift allowed |
| battery_energy_capacity | 100 | BESS energy capacity in MWh |
| battery_power_capacity | 25 | BESS maximum charge/discharge power in MW |
| battery_initial_state | 50 | Initial State of Charge (SoC) of the BESS in MWh |
| solar_pu_cost | 2.5 | Per-unit cost of solar generation in ₹/MWh |
| wind_pu_cost | 3.0 | Per-unit cost of wind generation in ₹/MWh |
| market_limit | 50 | Maximum power that can be purchased from/sold to IEX market per slot (MW) |
| Output Folder Name | Run_2026_June | Name of the output folder where generation schedules are saved |
| daily_slots | 24 | Number of time slots per day (24 for hourly, 96 for 15-minute intervals) |
| RE_forecasting | False | Boolean — whether to apply RE forecasting adjustments |

Important: List parameters must be entered with Python list syntax (e.g., [0, 0.05, 0.10]). Boolean values must be Python-compatible (True or False with capital first letter).

## 3.7 Generator Availability File (availability.xlsx)

This file defines the availability status of each generating unit for each time slot. It is used to model planned and unplanned outages.

| Specification | Details |
| --- | --- |
| Dimensions | Rows = num_slots, Columns = number of generating units (must match Generation_stack.xlsx) |
| Values | 1 = Generator available in that slot, 0 = Generator on outage (unavailable) |
| Column order | Must match the plant order in Generation_stack.xlsx exactly |

When a generator is marked 0 (unavailable), the model enforces that its scheduled generation is zero and it does not contribute to the must-run constraint for that slot.

# 4. Using the Application

## 4.1 Application Layout

The application has three main areas:

- Left Sidebar top: Application pages

- Left Sidebar bottom- Data Input: File upload panel and data initialization controls

- Main Panel: Analysis trigger button and run status indicators

3

2

1

## 4.2 Step-by-Step Workflow

### Step 1: Upload Input Files

In the left sidebar, upload each of the six required files using the file upload widgets. Files are accepted in CSV or Excel (.xlsx) format as specified in Section 3. All six files must be uploaded before proceeding.

### Step 2: Initialize Data

Once all files are uploaded, click the Process & Initialize Data button in the sidebar. The system will:

- Read and parse all uploaded files into memory

- Extract all configuration parameters from input_parameters.csv

- Initialize internal data structures for the optimization run

A green success message will confirm successful initialization. If any parameter parsing fails, an error message will indicate the issue. Common errors include incorrect Python list syntax in input_parameters.csv or mismatched column names.

### Step 3: Run the Analysis

Click the Start Analysis button in the main panel. The model will:

- Iterate over each flexibility level defined in the flexibility parameter

- For each flexibility level, run the LP optimization using PuLP to find the least-cost generation schedule

- Compute per-unit generation cost, optimised demand profile, battery dispatch, and market purchases for each scenario

- Store all results in session state for visualization

A progress bar shows completion status. Each flexibility scenario may take several seconds to several minutes to solve, depending on the number of time slots and generating units. For a full annual run (8760 hourly slots), expect approximately 2-5 minutes per scenario.

Note: Do not close or refresh the browser tab while the analysis is running. Results are held in memory and will be lost on page refresh.

### Step 4: Review Results

After the analysis completes, the dashboard displays overview of optimisation results:

- Bar chart: Total generation cost (₹) for each flexibility level

- Summary table: Total cost, absolute savings, and percentage savings versus the base case (0% flexibility) for each scenario

- flexibility analysis page, user can view demand heat maps, change in per unit cost due to up/down shift in demand (Incentive design), box charts for hourly change in demand for each month (Flexibility requirement), comparison between base demand and demand with different flexibility levels

- IEX page: provides IEX market price trends for last seven days, this helps user to understand recent trend and likely prices for upcoming days in planning DF/DR event

- Consumer analysis page helps in consumer meter data analysis and identifying consumers converging with peak cost periods

# 5. Understanding the Optimization Model

This section describes the mathematical optimization that drives the model, intended for energy planners and analysts who wish to understand the underlying logic.

## 5.1 Objective Function

The model minimizes the total variable generation cost across all time slots:

Minimize: Σ(t) Σ(p) [Schedule(t,p) × VariableCost(p)] + Σ(t) [MarketDrawl(t) × MarketRate(t)]

Where Schedule(t,p) is the generation level of plant p in slot t, and MarketDrawl(t) is the power purchased from IEX at slot t.

## 5.2 Decision Variables

| Variable | Description |
| --- | --- |
| Schedule(t, p) | Generation level of plant p at time slot t (MW) |
| OptimisedDemand(t) | Demand after applying flexibility/DR adjustments at slot t (MW) |
| MarketDrawl(t) | Power purchased from IEX market at slot t (MW, bounded by market_limit) |
| b_soc(t) | Battery State of Charge at slot t (MWh, bounded between 10% and 100% of capacity) |
| b_power(t) | Battery charge/discharge power at slot t (MW, negative = charging, positive = discharging) |
| TotalShift(t) | Absolute demand shift at slot t — used to constrain daily inconvenience cost |

## 5.3 Constraints

- Constraint 1 Ramp Rate Limits: Generation change between consecutive slots is bounded by Ramping_up and Ramping_down for each plant.

- Constraint 2 Technical Minimum: Each available generator must operate at or above its technical minimum (must-run level) in every slot.

- Constraint 3 Power Balance: Total scheduled generation equals optimised demand minus RE generation, minus market purchases, adjusted for battery net flow.

- Constraint 4 Generation Capacity: No plant can be scheduled above its rated capacity. Unavailable plants are locked to zero by the availability flag.

- Constraint 5 Market Limit: Market purchases and sales are bounded between -market_limit and +market_limit (negative = sell to market).

- Constraint 6 Battery Dynamics: Battery SoC follows physical charge/discharge dynamics; power is the change in SoC per slot.

- Constraint 7 Demand Flexibility Bounds: Optimised demand in each slot is bounded within ±flexibility × original demand.

- Constraint 8 Daily Energy Conservation: Total optimised demand each day equals total original demand for that day (no energy is destroyed, only shifted).

- Constraint 9 Daily Shift Limit: Total demand shift per day is bounded at 0.5% of daily demand, limiting consumer inconvenience.

## 5.4 Generation Cost Calculation

After optimization, total generation cost is calculated as:

Total Cost = (10⁻³) × (24/daily_slots) × [Σ(Plant Costs) + RE Costs + Market Costs]

The scaling factor (10⁻³ × 24/daily_slots) converts MW-slot values into MWh and then into consistent daily-equivalent cost units. Per-unit cost is then computed as total cost divided by total energy served.

Battery cost is calculated separately as 1.3× the energy charged (accounting for round-trip efficiency losses).

# 6. Output & Results

## 6.1 Generation Cost Chart

A bar chart displays total generation cost (₹) on the Y-axis against flexibility level (%) on the X-axis. The Y-axis is automatically scaled to the range of simulated costs to highlight differences. Hovering over any bar shows the exact flexibility level and total cost in Indian Rupees.

## 6.2 Summary Table

The summary table presents the following columns for each simulated flexibility level:

| Column | Unit | Description |
| --- | --- | --- |
| Flexibility | Decimal | The flexibility level simulated (0 = base case, 0.10 = 10%) |
| Total Cost (Million INR) | ₹ Million | Absolute total generation cost for the scenario |
| Total Cost Savings (Million INR) | ₹ Million | Savings relative to the base case (0% flexibility) |
| % Cost Savings | % | Percentage reduction in cost versus the base case |

## 6.3 Generation Schedule Files

For each flexibility scenario, the optimizer saves a CSV file containing the generation schedule for all plants at every time slot. These files are written to the output folder specified in Output Folder Name, under a subfolder called Generation schedules. File names include the flexibility level and a timestamp to avoid overwrites.

Output path format: <Output Folder Name>/Generation schedules/schedule_output_<flex>_<timestamp>.csv

Note: Ensure the OUTPUT_PATH in generation_cost2.py is set to a valid, writable directory on your system before running.

## 6.4 Base vs Optimised Demand Comparison

A line chart below compares the base demand (flex = 0) and the optimised demand for the selected flexibility level across all time slots. This chart is the clearest visualization of where demand has been shifted: the two lines diverge wherever the optimizer has rescheduled load.

## 6.5 DF/DR Savings Heatmap

The companion heatmap to the demand heatmap shows the per-unit cost saving from demand flexibility in ₹/kWh, structured the same way (hour-of-day × day-of-year). Bright cells indicate time slots where shifting demand away saved the most per unit. These are the most valuable slots for targeting DF/DR programmes — typically high-load afternoons or peak-tariff periods

## 6.6 High-Cost Period Analysis

The analyze_high_cost_contributors module (available for integration) identifies time slots that fall above a user-defined cost percentile (e.g., 75th percentile = top 25% most expensive hours). For each consumer meter, it computes:

| Metric | Description |
| --- | --- |
| Total Consumption (MWh) | Total energy consumed across all time slots |
| Peak Demand (MW) | Maximum demand recorded in any single slot |
| High-Cost Consumption (MWh) | Energy consumed specifically during high-cost periods |
| % During High Cost | Share of total consumption that occurs in high-cost periods |
| High-Cost Impact (₹) | Cost attributable to this meter during high-cost periods |
| Risk Score (0-100) | Composite score: 50% weight on consumption share, 30% on peak contribution, 20% on cost impact |

Meters are ranked by Risk Score (descending). High-scoring meters are the best candidates for demand response programmes.

# 7. Application Page Summary

The model is structured as a multi-page Streamlit application. The table below summarises all four pages, their entry points, and dependencies:

| Page | File | Purpose & Key Features |
| --- | --- | --- |
| Main Page | app.py | File upload, data initialization, LP optimization run, total cost bar chart, cost savings summary table |
| Flexibility Analysis | 1_Flexibility_analysis.py | Generation stack area chart, demand heatmap, base vs optimised demand comparison, DF/DR per-unit savings heatmap |
| IEX Market Data | 2_IEX.py | Live DAM and RTM market price trend charts for the past 7 days, benchmarking against model generation cost |
| Consumer Analysis | 3_consumer_analysis.py | Meter-level risk scoring for high-cost periods, consumer load vs cost correlation scatter, meter consumption heatmap |

Data flows from the main page to all subsequent pages through Streamlit session state. Pages 2, 3, and 4 will display warnings if the required session state variables are not present, and will stop execution until the main page analysis is completed.

## 7.1 Main page

Start Analysis

Click the analysis button after initialising the data. This will run the optimisation for all the flexibility level and shows result overview through charts and table.

7.1.1 Flexibility level wise generation cost

Bar chart shows total generation cost (₹) for each flexibility level.

7.1.2 Result summary

Table provides summary of results which includes total cost, absolute savings, and percentage savings versus the base case (0% flexibility) for each scenario

## 7.2 Flexibility Analysis

The Charts & Analysis page (1_flexibility_analysis.py) provides visual tools to explore the optimization results in detail. It is accessible only after a successful analysis run on the main page. If no optimization results are present in the session, the page displays a warning and stops loading.

Calculating Chart Parameters

Click the Calculate parameters for charts button. This step iterates over every time slot and flexibility level to compute:

| Derived Metric | Description |
| --- | --- |
| Demand Modulation | Difference between base-case optimised demand (flex = 0) and the demand at the selected flexibility level. A positive value means demand was shifted down (load reduction); negative means demand was shifted up. |
| DF Per-Unit Savings (₹/kWh) | Cost saved (or cost added) per unit of demand modulated. Computed only when modulation exceeds 1% of demand in that slot, avoiding division-by-near-zero noise. Zero is assigned for slots with negligible modulation. |

These derived dataframes are stored in session state as DF_pu_savings and Demand_modulation and persist for the duration of the browser session.

Flexibility Level Selector

A dropdown selector (Select Flexibility Level) appears after parameter calculation. All subsequent charts and heatmaps update to reflect the chosen flexibility level. The selector lists all flexibility levels from the input_parameters.csv flexibility list.

### 7.2.1 Generation Stack Area Chart

The left chart shows a stacked area chart of how total demand is served across time slots for the selected flexibility level:

| Trace | Description |
| --- | --- |
| Optimised Demand (×) | Scatter markers (white X) showing the optimised demand profile after DF/DR — the target the optimizer is meeting |
| Scheduled Generation (filled area) | Conventional plant dispatch output summed across all plants |
| Renewable Energy (stacked) | Solar + wind generation contribution |
| Battery Power (stacked) | Net battery contribution — negative battery power (charging) reduces the stack; positive (discharging) adds to it |
| Market Power (stacked) | Power purchased from the IEX market |

All traces use unified hover mode — hovering over any time slot shows all values simultaneously. The legend is positioned below the chart for readability.

### 7.2.2 Demand Heatmap

The right chart is a 2D heatmap of optimised demand intensity across the full analysis period:

- X-axis: Hour of day (or sub-hourly slot number within a day)

- Y-axis: Day of year (1 to 365)

- Color scale (Viridis): Dark purple = low demand, bright yellow = high demand

This view makes seasonal demand patterns immediately visible — summer afternoons with high air-conditioning load, winter morning peaks, and the impact of demand flexibility on load shape are all apparent at a glance.

## 7.2.3 Base vs Optimised Demand Comparison

A line chart below compares the base demand (flex = 0) and the optimised demand for the selected flexibility level across all time slots. This chart is the clearest visualization of where demand has been shifted: the two lines diverge wherever the optimizer has rescheduled load

## 7.2.4 DF/DR Savings Heatmap

The companion heatmap to the demand heatmap shows the per-unit cost saving from demand flexibility in ₹/kWh, structured the same way (hour-of-day × day-of-year). Bright cells indicate time slots where shifting demand away saved the most per unit. These are the most valuable slots for targeting DF/DR programmes — typically high-load afternoons or peak-tariff periods.

Tip: Export the DF savings heatmap data to identify the top 10% of hours where demand flexibility provides the highest cost benefit. These hours should be prioritised in field DR programmes and consumer incentive designs.

## 7.3 IEX Market MCP Data Trend

The IEX Market MCP Data Trend page (2_IEX.py) provides live market price benchmarking by fetching and displaying the Market Clearing Price (MCP) from the Indian Energy Exchange for the Day-Ahead Market (DAM) and Real-Time Market (RTM) over the past week.

This page allows DISCOM analysts to compare their internal generation cost (from the optimization model) against current market prices. If the market price is lower than the per-unit generation cost, it may be more economical to procure additional power from the exchange rather than dispatching expensive conventional plants.

Loading Market Data

Click Load IEX market data. The application automatically determines the date range:

| Market | Date Range Fetched |
| --- | --- |
| DAM (Day-Ahead Market) | Last 7 days from today's date |
| RTM (Real-Time Market) | 7 days ending yesterday (slight lag due to RTM settlement timing) |

Data is scraped in 15-minute intervals directly from the IEX website using the built-in iex_data.py scraper. A 1-second delay between daily requests is enforced to avoid server overload. The scraper fetches the market snapshot page for each date and parses the main data table.

Note: An active internet connection is required. If the IEX website is unavailable or its page structure has changed, the scraper may return no data. In that case, use the pre-downloaded test file (iex_dam_test_20260413_20260414.csv) or manually prepare a market_rate.xlsx file.

## 7.3.1 DAM MCP Trend Chart

Under the DAM MCP TREND tab, a multi-line chart shows the Day-Ahead Market clearing price for each of the past 7 days. Each day is plotted as a separate-coloured line:

- X-axis: 15-minute time blocks across the day

- Y-axis: MCP in ₹/kWh

- Each line represents one calendar date; the legend identifies dates

- Hover tooltip shows date, time block, and exact MCP value

The chart uses unified hover mode, allowing comparison of prices across all dates at a given time block simultaneously. This is particularly useful for identifying recurring expensive time windows (e.g., evening peaks between 18:00 and 22:00).

## 7.3.2 RTM MCP Trend Chart

Under the RTM MCP TREND tab, the same multi-line chart format is applied to Real-Time Market prices. RTM prices are more volatile than DAM prices and reflect short-term supply-demand imbalances. Very high RTM prices (spikes) indicate periods of grid stress where demand reduction would have maximum value.

Column indexing note: The DAM data uses column index 2 for time and column index 7 for MCP. The RTM data uses column index 3 for time and column index 8 for MCP. These offsets reflect the different table structures on the IEX website. If the IEX website updates its table format, these column indices may need adjustment in 2_IEX.py.

## 7.4 Consumer Demand Analysis for High-Cost Periods

The Consumer Demand Analysis page (3_consumer_analysis.py) provides granular, meter-level analysis to identify which consumers contribute most to high-cost generation periods. This is the primary tool for designing targeted Demand Response programmes.

This page requires two things to be available in the session:

- A completed optimization run (pu_gen_cost_flex must be present in session state from the main page)

- A meter data CSV file containing hourly or sub-hourly consumption readings for individual consumer meters

Meter Data File Format

The meter data file must be a CSV where:

| Specification | Requirement |
| --- | --- |
| Rows | Must match num_slots exactly (e.g., 8760 for annual hourly data) |
| Columns | Each column represents one consumer meter. Column header = meter ID or meter number |
| Values | Consumption in kW (or MW — be consistent with the units used in pu_gen_cost) |
| File location | Must be accessible from the machine running the Streamlit server. Enter the full absolute path. |

Example path format (Windows): C:/Users/YourName/data/meter_data.csv

Example path format (Linux/Mac): /home/username/data/meter_data.csv

Loading Meter Data

Enter the full file path in the text input field and click Load Meter Data. The file is read into session state as consumer_data. A success message confirms loading. Only .csv files are accepted — if a non-CSV path is entered, a warning is shown.

High-Cost Threshold Setting

The percentile input (Enter the threshold percentile for PU gen cost) defines what constitutes a high-cost period. The default value is 10.0, meaning the analysis will flag the top 10% most expensive generation hours (i.e., the 90th percentile threshold and above).

Adjusting this value:

- Lower percentile (e.g., 5) → fewer hours flagged as high-cost, only the most extreme peaks

- Higher percentile (e.g., 25) → more hours included, broader identification of expensive periods

The analysis runs automatically once both meter data and optimization results are loaded. Results are stored in Consumer_data_results in session state.

## 7.4.1 Consumer Analysis Table

The full results table ranks all consumer meters by their Risk Score (descending). Each row represents one meter with the following columns:

| Column | Unit | Description |
| --- | --- | --- |
| Meter | — | Meter ID or column header from the input file |
| Total_Consumption_MWh | MWh | Total energy consumed across all time slots |
| Peak_Demand_MW | MW | Maximum demand recorded in any single slot |
| Avg_Consumption_MW | MW | Average demand across all slots |
| High_Cost_Consumption_MWh | MWh | Energy consumed specifically in high-cost slots |
| Pct_During_High_Cost_% | % | Percentage of this meter's total consumption that falls in high-cost periods |
| Pct_of_Peak_in_High_Cost_% | % | The meter's peak demand in high-cost slots as a percentage of its overall peak |
| High_Cost_Avg_MW | MW | Average consumption during high-cost slots only |
| High_Cost_Peak_MW | MW | Maximum demand during high-cost slots |
| Total_Cost_Impact_₹ | ₹ | Total cost attributable to this meter across all time slots |
| High_Cost_Impact_₹ | ₹ | Cost attributable to this meter specifically in high-cost slots |
| Risk_Score | 0–100 | Composite score: 50% consumption share weight + 30% peak share weight + 20% cost impact weight |

Meter Selection and Drill-Down

A dropdown (Consumer meter) allows selection of any individual meter for detailed visual analysis. Two charts are shown:

### 7.4.2 Correlation: Consumer Load vs PU Generation Cost

A scatter plot with consumer demand on the X-axis and per-unit generation cost on the Y-axis. Each point is one time slot. A positive correlation (points clustering upper-right) indicates this consumer tends to draw high power precisely during expensive generation periods — making them a high-priority DR candidate. A flat or negative correlation suggests the consumer's load profile is not aligned with peak cost periods.

### 7.4.3 Meter Data Heatmap

A 2D heatmap of the selected meter's consumption across the year:

- X-axis: Hour of day (time slot within the day)

- Y-axis: Day of year

- Color: Consumption intensity in kW (Viridis scale)

This heatmap reveals the meter's habitual consumption patterns — whether load is concentrated in mornings, evenings, or seasonally variable. Comparing this with the DF savings heatmap from Page 2 immediately identifies whether this consumer's peak hours align with the highest-value DR windows.

Note: The consumer analysis page also exports total_cost_flex and opt_demand to fixed Excel paths (hardcoded to a local desktop path in the current version). Update these paths in 3_consumer_analysis.py before deployment on a new machine.

# 8. Troubleshooting

## 8.1 Common Errors and Solutions

| Error / Symptom | Likely Cause | Resolution |
| --- | --- | --- |
| 'Error parsing files' on initialization | Wrong column names, missing parameters, or incorrect list syntax in input_parameters.csv | Check column names match exactly. Ensure lists use Python syntax: [0, 0.05, 0.10] |
| LP solver returns 'Infeasible' | Technical minimum exceeds net demand, or flexibility bounds are too tight | Check that Σ(Technical_min) < min(net_demand). Widen flexibility bounds or check RE profiles |
| Progress bar stuck or very slow | Large number of time slots (e.g., 35040 for 15-min annual) with many plants | Reduce num_slots for testing. Run overnight for full annual runs. |
| FileNotFoundError for output CSV | OUTPUT_PATH in generation_cost2.py does not exist or is read-only | Edit OUTPUT_PATH in generation_cost2.py to a valid writable path on your system |
| IEX scraper returns no data | IEX website structure changed or network access blocked | Check internet connectivity. Try fetching the URL manually in a browser to confirm availability |
| Session state lost after page refresh | Streamlit session state is in-memory and does not persist across browser refreshes | Re-upload files and re-run analysis. Do not refresh during a run. |
| 'KeyError: rate' in market rate loading | market_rate.xlsx does not have a column named 'rate' | Rename the rate column in market_rate.xlsx to exactly 'rate' (lowercase) |

## 8.2 Performance Tips

- For initial testing, use a short period (e.g., one week = 168 hourly slots) to verify results before running a full annual analysis.

- Reduce the number of flexibility levels in the flexibility list during testing (e.g., [0, 0.10] only).

- Ensure the availability.xlsx file has no blank cells — replace all blanks with 1 (available) or 0 (unavailable).

- Use a machine with at least 8 GB RAM for full annual runs with more than 5 generating plants.

# 9. Updated File Structure

| File | Type | Purpose |
| --- | --- | --- |
| app.py | Python | Main page — file upload, initialization, LP optimization, cost summary |
| 1_charts.py | Python | Page 2 — generation stack area chart, demand heatmap, DF savings heatmap |
| 2_IEX.py | Python | Page 3 — IEX DAM/RTM live market price scraping and trend charts |
| 3_consumer_analysis.py | Python | Page 4 — consumer meter risk scoring, correlation scatter, load heatmap |
| generation_cost2.py | Python | Core LP optimization engine |
| generation_cost.py | Python | Earlier optimizer version (reference only) |
| analyze_high_cost_contributors.py | Python | Consumer meter risk analysis module |
| iex_data.py | Python | IEX market data scraper |
| month_info.py | Python | Calendar utility |
| requirements.txt | Text | Python package dependencies |

# 10. Glossary

| Term | Definition |
| --- | --- |
| DISCOM | Distribution Company — the licensed entity responsible for electricity distribution and supply to end consumers in a given geography in India |
| DSM | Demand-Side Management — a set of programmes aimed at influencing consumer electricity usage patterns |
| DF (Demand Flexibility) | The ability to shift demand across time slots within a day while keeping daily consumption constant |
| DR (Demand Response) | Temporary load curtailment or reduction in response to a grid signal or economic incentive |
| LP (Linear Programming) | A mathematical optimization technique that finds the minimum-cost solution subject to linear constraints |
| PuLP | An open-source Python library for formulating and solving linear programming problems |
| PPA (Power Purchase Agreement) | A contract between a DISCOM and a power generator specifying capacity, cost, and supply terms |
| BESS (Battery Energy Storage System) | A grid-scale battery used to store energy during low-cost periods and discharge during high-cost periods |
| IEX | Indian Energy Exchange — India's electricity spot market |
| DAM | Day-Ahead Market — IEX market segment where power for the next day is traded one day in advance |
| RTM | Real-Time Market — IEX market segment where power is traded close to the actual delivery hour |
| MCP | Market Clearing Price — the equilibrium price at which buy and sell bids are matched on the IEX exchange |
| Technical Minimum | The lowest generation level at which a thermal or gas plant can operate stably |
| Ramping Rate | The maximum rate at which a generator can increase or decrease output between consecutive time slots |
| Net Demand | Gross consumer demand minus renewable energy generation |
| SoC (State of Charge) | The current energy stored in the battery, in MWh |
| Per-Unit Cost | Total generation cost divided by total energy served (₹/kWh or ₹/MWh) |
| Risk Score | A composite 0–100 score indicating how much a consumer meter contributes to high-cost generation periods |
| Demand Modulation | The difference between base-case and flexibility-adjusted optimised demand at each time slot |
| DF Savings (₹/kWh) | The cost reduction per unit of demand modulation — the value signal for incentivising demand shift |

# 11. Support & Maintenance

For technical issues, model configuration queries, or enhancement requests, please contact the model development team. When reporting issues, include:

- The exact error message or symptom observed

- Which page of the application the error occurred on

- The input_parameters.csv values used in that run

- The number of time slots and generating plants in the model

For large-scale deployment or customization — including integration with SCADA systems, real-time meter data feeds, or multi-DISCOM portfolio models — dedicated technical support is available separately.

This manual covers Version 1.0 of the Demand Flexibility Analysis Model. Features and interfaces may change in future versions.