DEMAND FLEXIBILITY

ANALYSIS MODEL

User Manual

Developed by: MP Ensystems Advisory Pvt Ltd

Version 2.0

June 2026

# Table of Contents

Table of Contents	2

1. Introduction	5

1.1 Purpose	5

1.2 Background: Demand Flexibility for DISCOMs	5

1.3 Key Features	5

2. System Requirements & Installation	7

2.1 Software Prerequisites	7

2.2 Installation Steps	7

Online Access (No Installation Required)	7

3. Input Data Files	8

3.1 File Overview	8

3.2 Generation Stack File (Generation_stack.xlsx)	8

3.3 Demand Data File (demand.csv)	8

3.4 Market Rate File (market_rate.xlsx)	9

3.5 Renewable Energy File (RE.xlsx)	9

3.6 Generator Availability File (availability.xlsx)	9

4. Using the Application	10

4.1 Application Layout	10

4.2 Step-by-Step Workflow	10

Step 1: Upload Input Files	10

Step 2: Configure Model Parameters	10

Step 3: Initialize Data	10

Step 4: Run the Analysis	11

Step 5: Review Results	11

5. Understanding the Optimization Model	12

5.1 Objective Function	12

5.2 Decision Variables	12

5.3 Constraints	12

5.4 Generation Cost Calculation	12

6. Output & Results	13

6.1 Generation Cost Chart	13

6.2 Summary Table	13

6.3 Generation Schedule Files	13

6.4 Downloadable Results	13

7. Application Page Guide	14

7.1 Main Page	14

7.1.1 Model Parameter Panel	14

7.1.2 Flexibility Level-wise Generation Cost	15

7.1.3 Result Summary Table	15

7.2 RE Generation Forecasting Page	15

Stage 1: Configure Plants	15

Stage 2: Upload Plant Data	15

Stage 3: Train & Forecast	15

RE Forecast Output	15

7.3 ANN Time-Series Forecasting Page	16

Input Data Format	16

Stage 2: Configure & Train	16

Training Results & Test Charts	17

Stage 3: Year-Ahead Forecast	17

7.4 Flexibility Analysis Page	17

7.4.1 Generation Stack Area Chart	17

7.4.2 Demand Heatmap	18

7.4.3 Base vs Optimised Demand Comparison	18

7.4.4 DF/DR Savings Heatmap	18

7.5 Consumer Demand Analysis Page	18

Meter Data Upload	18

High-Cost Threshold	18

7.5.1 Consumer Risk Analysis Table	18

7.5.2 Consumer Load vs PU Cost Scatter	18

7.5.3 Meter Consumption Heatmap	18

7.6 Generator utilisation	19

7.7 IEX Market MCP Data Trend Page	19

Why Live Fetch May Fail on the Online Version	19

7.7.1 DAM MCP Trend Chart	19

7.7.2 RTM MCP Trend Chart	19

8. Troubleshooting	20

8.1 Common Errors and Solutions	20

8.2 Performance Tips	20

9. Model File Structure	21

10. Glossary	22

11. Support & Maintenance	24

# 1. Introduction

The Demand Flexibility Analysis Model is a web-based analytical tool developed to help Indian Electricity Distribution Companies (DISCOMs) evaluate the economic and operational impact of demand-side management (DSM) strategies on power procurement and generation costs. The platform is comprehensive energy planning suite incorporating LP-based generation cost optimization, renewable energy forecasting, year-ahead demand and market price forecasting, and live IEX market data integration.

## 1.1 Purpose

This tool enables DISCOM planners and energy managers to:

- Simulate how adjusting demand shapes affects generation cost

- Quantify change in generation cost per unit change in demand (up or down), usable for estimating DR incentives

- Compare generation costs across multiple demand flexibility levels in a single run

- Forecast hourly solar and wind generation using LSTM models trained on historical weather data

- Forecast year-ahead demand and IEX market prices using a custom ANN model

- Identify high-cost time periods and pinpoint consumer meters that contribute to peak costs

- Evaluate the benefit of battery energy storage dispatch in reducing peak costs

## 1.2 Background: Demand Flexibility for DISCOMs

Demand Flexibility (DF) refers to the ability to shift, reduce, or increase electrical consumption in response to grid signals or economic incentives. For DISCOMs, this translates into:

- Demand Flexibility (DF): Shifting consumption across time slots within a day while keeping daily totals constant.

- Demand Response (DR): Temporary curtailment of load in response to high-cost or high-stress grid periods.

By modelling different flexibility levels (e.g., 5%, 10%, 15% of peak demand that can be shifted), DISCOMs can quantify savings from DF programmes before committing resources to field implementation.

## 1.3 Key Features

- LP-based generation cost optimization using PuLP solver

- Multi-plant generation scheduling with ramping, technical minimum, and capacity constraints

- Battery Energy Storage System (BESS) dispatch modelling

- IEX market purchase integration

- LSTM-based RE generation forecasting (solar + wind) per plant

- ANN-based year-ahead time-series forecasting for demand and market prices

- Interactive model parameter configuration directly in the UI

- High-cost period identification and consumer meter contribution analysis

- Interactive Streamlit dashboard with Plotly charts, downloadable results, and cross-page session integration

# 2. System Requirements & Installation

## 2.1 Software Prerequisites

| Requirement | Details |
| --- | --- |
| Python | Version 3.9 or higher |
| Streamlit | >=1.35.0 (web app framework) |
| PuLP | >=2.7.0 (LP optimization solver) |
| Pandas / NumPy | >=2.0.0 / >=1.24.0 (data handling) |
| Plotly | >=5.18.0 (interactive charts) |
| TensorFlow | >=2.12.0 (LSTM RE forecasting) |
| pvlib | >=0.10.0 (solar irradiance transposition) |
| openpyxl / xlrd | >=3.1.0 / >=2.0.0 (Excel file support) |
| Requests / lxml | >=2.31.0 (IEX data scraping) |
| SciPy / pytz | >=1.10.0 / >=2023.3 |

## 2.2 Installation Steps

- Ensure Python 3.9+ is installed on your system.

- Download or clone the model files to a local directory.

- Open a terminal/command prompt and navigate to the model directory.

- Install all dependencies by running:

pip install -r requirements.txt

- Launch the application by running:

streamlit run app.py

- The application will open in your default browser. If not, navigate to http://localhost:8501

### Online Access (No Installation Required)

The model is also hosted online at:

https://demandflexibilityapp.streamlit.app/

No Python or setup required — open the link in any modern browser. Note that large optimization runs may take longer on the hosted version depending on server load. Handle sensitive or proprietary input data with discretion when using the hosted version.

# 3. Input Data Files

The model requires five input files uploaded through the sidebar. All five files must be uploaded before launching an analysis run.

## 3.1 File Overview

| File Name | Format | Description |
| --- | --- | --- |
| Generation_stack.xlsx | Excel (.xlsx) | Generator portfolio: capacity, costs, ramp rates, technical minimums |
| demand.csv | CSV | Hourly or sub-hourly projected demand (MW) for the analysis period |
| market_rate.xlsx | Excel (.xlsx) | IEX market purchase rates (₹/MWh) for each time slot |
| RE.xlsx | Excel (.xlsx) | Renewable energy generation profiles — Solar and Wind (MW) |
| availability.xlsx | Excel (.xlsx) | Generator availability flags (1=available, 0=unavailable) per time slot |

## 3.2 Generation Stack File (Generation_stack.xlsx)

Each row represents one generating unit.

| Column Name | Unit | Description |
| --- | --- | --- |
| Capacity | MW | Installed/contractual capacity of the generating unit |
| Technical_min | MW | Minimum generation level the unit must operate at (must-run) |
| Fixed cost | ₹/hr | Fixed operating cost per hour |
| Variable cost | ₹/MWh | Variable generation cost — used in the LP objective function |
| Ramping_up | MW/slot | Maximum rate of upward change between consecutive time slots |
| Ramping down | MW/slot | Maximum rate of downward change (negative value) |

## 3.3 Demand Data File (demand.csv)

| Parameter | Specification |
| --- | --- |
| Column | demand (MW) |
| Rows | Equal to num_slots (e.g., 8760 for hourly annual, 35040 for 15-min annual) |
| Data type | Numeric, positive values only |
| Note | Can be replaced by output from the ANN Forecasting module — see Section 7.5 |

## 3.4 Market Rate File (market_rate.xlsx)

| Parameter | Specification |
| --- | --- |
| Column name | rate |
| Unit | ₹/MWh |
| Rows | Must match num_slots exactly |
| Typical source | IEX DAM/RTM data (fetched via IEX page or uploaded manually) |

## 3.5 Renewable Energy File (RE.xlsx)

| Column Name | Unit | Description |
| --- | --- | --- |
| Solar | MW | Solar PV generation profile for each time slot |
| Wind | MW | Wind generation profile for each time slot |

RE.xlsx can be replaced by the output of the RE Forecasting module (Page 5). After running the RE forecast, click 'Use this forecast as RE input in the DF Model' to load it directly into the session without re-uploading a file.

## 3.6 Generator Availability File (availability.xlsx)

| Specification | Details |
| --- | --- |
| Dimensions | Rows = num_slots, Columns = number of generating units (must match Generation_stack.xlsx) |
| Values | 1 = Generator available in that slot, 0 = Generator on outage |
| Column order | Must match the plant order in Generation_stack.xlsx exactly |

# 4. Using the Application

## 4.1 Application Layout

The application has three main areas:

- Left Sidebar top: Application page navigation (six pages)

- Left Sidebar bottom — Data Input: File upload panel and data initialization controls

- Main Panel: Model parameter configuration, analysis trigger button, results, and charts

## 4.2 Step-by-Step Workflow

### Step 1: Upload Input Files

In the left sidebar, upload the five required files (Generation Stack, Demand Data, Market Rates, Renewable Energy, Generator Availability). The input_parameters.csv file is no longer required.

### Step 2: Configure Model Parameters

On the main page, a three-column parameter panel appears before the Start Analysis button. All model parameters are configured here — no CSV file needed. The parameters are grouped into three columns:

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Flexibility level – Minimum | RE costs (solar ₹/MWh, wind ₹/MWh) | Battery energy capacity (MWh) |
| Flexibility level – Maximum | IEX market purchase/sale limit (MW) | Battery power capacity (MW) |
| Step for increasing flexibility from min to max | RE forecast toggle (on/off) | Battery initial SoC (MWh) |
|  |  | Time resolution (slots per day: 24/48/96) |
| Max daily shift constraint (fraction of daily demand) |  | Output folder name (text) |

Flexibility levels are built from three sliders (min, max, step) that automatically generate the list of scenarios to simulate. The base case (0% flexibility) is always prepended. A preview caption shows the exact list that will be used.

The RE forecast toggle integrates with Page 5 (RE Forecasting). If the toggle is ON and a forecast has already been generated on Page 5, the model uses that forecast instead of the uploaded RE.xlsx — no need to re-upload a file.

### Step 3: Initialize Data

Click Process & Initialize Data in the sidebar. The system reads all uploaded files and saves all UI parameter values into session state. A green success message confirms initialization.

### Step 4: Run the Analysis

Click Start Analysis. The model iterates over each flexibility level, runs the LP optimization for each, and stores results in session state. A progress bar shows completion. For a full annual run (8760 hourly slots), expect 2–5 minutes per scenario.

Do not close or refresh the browser tab while the analysis is running. Results are held in memory and will be lost on page refresh.

### Step 5: Review Results

After completion, the main page shows the generation cost bar chart and cost savings summary table. Navigate to other pages for detailed charts, IEX benchmarking, RE forecasting, ANN forecasting, and consumer analysis.

# 5. Understanding the Optimization Model

## 5.1 Objective Function

Minimize: Σ(t) Σ(p) [Schedule(t,p) × VariableCost(p)] + Σ(t) [MarketDrawl(t) × MarketRate(t)]

## 5.2 Decision Variables

| Variable | Description |
| --- | --- |
| Schedule (t, p) | Generation level of plant p at time slot t (MW) |
| OptimisedDemand(t) | Demand after applying flexibility/DR adjustments at slot t (MW) |
| MarketDrawl(t) | Power purchased from IEX market at slot t (MW, bounded by market_limit) |
| b_soc(t) | Battery State of Charge at slot t (MWh, bounded between 10% and 100% of capacity) |
| b_power(t) | Battery charge/discharge power at slot t (MW) |
| TotalShift(t) | Absolute demand shift at slot t — used to constrain daily inconvenience cost |

## 5.3 Constraints

- Constraint 1 Ramp Rate Limits: Generation change bounded by Ramping_up and Ramping_down for each plant.

- Constraint 2 Technical Minimum: Each available generator must operate at or above its technical minimum.

- Constraint 3 Power Balance: Total scheduled generation equals optimised demand minus RE, minus market purchases, adjusted for battery net flow.

- Constraint 4 Generation Capacity: No plant can be scheduled above its rated capacity.

- Constraint 5 Market Limit: Market purchases/sales bounded between ±market_limit (configured in the UI).

- Constraint 6 Battery Dynamics: Battery SoC follows physical charge/discharge dynamics.

- Constraint 7 Demand Flexibility Bounds: Optimised demand bounded within ±flexibility × original demand.

- Constraint 8 Daily Energy Conservation: Total optimised demand each day equals total original demand (energy shifted, not lost).

- Constraint 9 Daily Shift Limit: Total demand shift per day bounded by max_daily_shift (configured in UI, default 0.5%).

## 5.4 Generation Cost Calculation

Total Cost = (10⁻³) × (24/daily_slots) × [Σ(Plant Costs) + RE Costs + Market Costs]

# 6. Output & Results

## 6.1 Generation Cost Chart

Bar chart of total generation cost (₹) vs flexibility level (%). Y-axis auto-scaled to highlight cost differences. Hover shows exact values.

## 6.2 Summary Table

| Column | Unit | Description |
| --- | --- | --- |
| Flexibility | Decimal | Flexibility level (0 = base case, 0.10 = 10%) |
| Total Cost (Million INR) | ₹ Million | Absolute total generation cost for the scenario |
| Total Cost Savings (Million INR) | ₹ Million | Savings relative to the base case (0% flexibility) |
| % Cost Savings | % | Percentage reduction in cost versus the base case |

## 6.3 Generation Schedule Files

For each flexibility scenario, a CSV with the full generation schedule is saved to the Output Folder Name specified in the UI parameters, under a subfolder called Generation schedules. File names include flexibility level and timestamp.

Ensure the OUTPUT_PATH in generation_cost2.py is set to a valid, writable directory on your system before running.

## 6.4 Downloadable Results

The RE Forecasting page and ANN Forecasting page each provide download buttons for their outputs in both CSV and Excel formats. The ANN forecast Excel file also includes a monthly summary sheet. All downloads are generated server-side and work on both local and online versions.

# 7. Application Page Guide

The model is structured as a seven-page Streamlit application. All pages share session state — results from the main page are available on all other pages without re-running.

| Page | File | Purpose & Key Features |
| --- | --- | --- |
| Main Page | app.py | File upload, UI parameter config, LP optimization, cost bar chart, cost savings table |
| RE Forecasting | 0.RE_forecast.py | LSTM-based solar and wind generation forecast per plant, aggregated RE.xlsx output |
| ANN Forecasting | 1.Year_ahead_forecasting.py | Year-ahead demand/price forecasting using custom numpy ANN, cost curve, test chart, monthly summary |
| Flexibility Analysis | 2.charts.py | Generation stack chart, demand heatmap, base vs optimised demand, DF/DR savings heatmap |
| Consumer Analysis | 3.consumer_analysis.py | Meter risk scoring, consumer load vs cost scatter, meter heatmap |
| Generation scheduling | 4.generator_utilisation | Generator wise contribution to total demand, load factor |
| IEX Market Data | 5.IEX.py | Live fetch OR manual CSV upload for DAM/RTM MCP trend charts |

## 7.1 Main Page

### 7.1.1 Model Parameter Panel

A three-column interactive panel on the main page for parameter configuration:

- Flexibility levels: min/max/step sliders auto-generate the scenario list. Base case (0%) always included.

- Time resolution: dropdown for 24 (hourly), 48 (30-min), or 96 (15-min) slots per day.

- RE costs: solar and wind per-unit costs in ₹/MWh.

- Market limit: maximum MW that can be bought or sold on IEX per time slot.

- Battery parameters: energy capacity (MWh), power capacity (MW), initial SoC (MWh). Initial SoC is bounded by energy capacity automatically.

- Max daily shift: fraction of daily demand that can be shifted — controls consumer inconvenience. Default 0.005 (0.5%).

- RE forecast toggle: when ON, uses the forecast from Page 5 instead of the uploaded RE.xlsx.

- Output folder name: subfolder for saving generation schedule CSVs.

### 7.1.2 Flexibility Level-wise Generation Cost

Bar chart showing total generation cost (₹) for each simulated flexibility level

### 7.1.3 Result Summary Table

Table showing total cost, absolute savings, and percentage savings versus the base case for each flexibility scenario.

## 7.2 RE Generation Forecasting Page

LSTM-based forecasting of hourly solar and wind generation for each plant, aggregated into a single RE.xlsx ready for use in the DF model. The page operates in three sequential stages:

### Stage 1: Configure Plants

Set the number of plants (1–20), training start/end years, and forecast target year. These apply to all plants.

### Stage 2: Upload Plant Data

Each plant requires three files:

| File | Required Columns | Notes |
| --- | --- | --- |
| Weather CSV (*Plant.csv) | Timestamp, Temperature, GHI, Wind Speed | Hourly rows, Timestamp parseable as datetime |
| Solar Info CSV (Info_Solar.csv) | Parameter, Value | Contains: Latitude, Longitude, Pnom (kW), nDRT, alpha, gamma, A_PV |
| Wind Info CSV (Info_Wind.csv) | Parameter, Value | Contains: Latitude, Longitude, Prtdwn (kW), Vctin, Vrtdwn, Vctout |

Each plant can be typed as Solar, Wind, or Solar + Wind. The Solar Info and Wind Info uploaders are automatically disabled for plant types that do not require them.

### Stage 3: Train & Forecast

Click Run Forecast. For each plant, three LSTM models are trained (Temperature, GHI, Wind Speed) on historical data, then used to forecast the target year. Advanced settings (LSTM units, epochs, batch size, look-back steps) are available in a collapsible expander — defaults match the reference notebook.

A dual progress bar shows overall progress (per plant) and per-feature training progress. Expect 2–5 minutes per plant depending on data size and server load.

### RE Forecast Output

| Output | Description |
| --- | --- |
| Aggregated generation chart | Full-year hourly Solar and Wind (MW) overlaid on one chart |
| Generation heatmaps | Solar (YlOrRd) and Wind (Blues) heatmaps — Hour of day × Day of year |
| Monthly summary table | Total, peak, and mean generation by month (auto-generated for 8760-row outputs) |
| KPI metrics | Total Solar MWh, Total Wind MWh, Peak Solar MW, Peak Wind MW |
| RE.xlsx download | Solar + Wind columns in MW, ready to upload to the DF model |
| Forecast CSV download | Timestamped hourly forecast |
| Direct session load button | Loads forecast into session state — main page picks it up without file re-upload |

If only solar plants are configured, wind output defaults to zero (and vice versa). The RE.xlsx always contains both Solar and Wind columns so it remains compatible with the DF model.

## 7.3 ANN Time-Series Forecasting Page

A generic year-ahead forecasting tool using a custom numpy-based Artificial Neural Network (ANN). Suitable for any time-series data — state electricity demand, IEX market prices, or custom datasets. Three preset use cases are provided with optimised default parameters:

| Use Case | Default Architecture | Typical Application |
| --- | --- | --- |
| State Electricity Demand | 5→5→1 | Year-ahead hourly state demand using 5 years of historical sorted hourly values |
| IEX Market Price (MCP) | 2→2→1 | Year-ahead DAM MCP forecast using 2 years of sorted hourly price data |
| Custom / Generic | 4→4→1 | Any time-series dataset in the standard column format |

### Input Data Format

Upload a CSV or Excel file where each column is one year (or one input feature) of sorted historical data, and the last column is the target variable to forecast. Each row is one time slot. Example: for a 5-year state demand forecast, the file has 6 columns (5 input years + 1 target year) and 8760 rows.

### Stage 2: Configure & Train

Configure the model using the three-column parameter panel:

- Network architecture: comma-separated layer sizes (e.g. 5,8,4,1). First value auto-corrected to match input features; last value must be 1.

- Training split: percentage of data used for training (default 70%); remainder is the test set.

- Learning rate and iteration count: adjust and retrain without re-uploading data.

- Cost curve smoothing: plot one cost point every N iterations to reduce noise.

Training runs with a 20-step chunked progress bar. The ANN uses He initialisation for ReLU hidden layers and a linear output layer, matching the reference notebook architecture.

### Training Results & Test Charts

After training, five sections are displayed:

- KPI row: Final MSE, Test nRMSE (%), Test sMAPE (%), Test R², Test MAE

- Cost curve: MSE loss vs iteration number, smoothed, with architecture and hyperparameter caption

- Test set actual vs predicted: both lines overlaid with train/test split shaded region

- Full dataset overlay: historical actual + model output across train and test regions, with vertical split line

- Detailed metrics table: training set and test set metrics side by side (RMSE, nRMSE, sMAPE, MAE, R²)

- Actual vs Predicted scatter (collapsible): test set scatter with perfect-fit diagonal line

### Stage 3: Year-Ahead Forecast

Upload a file containing only the input feature columns (no target column) for the forecast period. The model normalises using training data's max values (no data leakage). Output includes:

- Forecast chart: overlays full historical series + forecast continuation with 'Forecast starts' marker

- Monthly summary table: auto-generated for 8760-row forecasts (total, peak, mean, min per month)

- Download buttons: CSV and Excel (Excel includes monthly summary on second sheet)

- 'Use as Demand input in DF Model' button: pushes forecast directly into session state — main page uses it without file re-upload

The ANN model is implemented in pure numpy — no TensorFlow or scikit-learn required for this module.

## 7.4 Flexibility Analysis Page

Accessible only after a successful main-page optimization run. Click Calculate parameters for charts to compute derived metrics:

| Derived Metric | Description |
| --- | --- |
| Demand Modulation | Difference between base-case and selected-flexibility optimised demand. Positive = demand shifted down; negative = shifted up. |
| DF Per-Unit Savings (₹/kWh) | Cost saved per unit of demand modulated. Computed only when modulation >1% of demand to avoid near-zero division noise. |

### 7.4.1 Generation Stack Area Chart

Stacked area chart showing how demand is served for the selected flexibility level. Traces: Optimised Demand (scatter markers), Scheduled Generation, Renewable Energy, Battery Power, Market Power. All use unified hover mode.

### 7.4.2 Demand Heatmap

2D heatmap of optimised demand intensity (Hour of day × Day of year, Viridis scale). Shows seasonal and daily demand patterns and the effect of demand flexibility on load shape.

### 7.4.3 Base vs Optimised Demand Comparison

Line chart overlaying base demand (flex=0) and optimised demand for the selected flexibility level. Lines diverge wherever the optimizer rescheduled load.

### 7.4.4 DF/DR Savings Heatmap

Per-unit cost saving from demand flexibility in ₹/kWh (Hour of day × Day of year). Bright cells = highest-value time slots for targeting DR programmes.

Compare this heatmap with the consumer meter heatmap on Page 4 to identify consumers whose peak hours align with the highest-value DR windows.

## 7.5 Consumer Demand Analysis Page

Meter-level analysis to identify consumers contributing most to high-cost generation periods. Requires a completed optimization run from the main page.

### Meter Data Upload

Upload a CSV file directly using the file uploader widget. Each column = one consumer meter (ID or number as header). Each row = one time slot. Values in kW. Number of rows must match num_slots.

### High-Cost Threshold

The percentile input defines what constitutes a high-cost period. Default 10.0 means the top 10% most expensive hours. Lower values flag only extreme peaks; higher values flag broader expensive periods.

### 7.5.1 Consumer Risk Analysis Table

Ranks all consumer meters by Risk Score (descending). Risk Score = 50% consumption share weight + 30% peak share weight + 20% cost impact weight. Columns include Total Consumption, Peak Demand, High-Cost Consumption, High-Cost Impact (₹), and Risk Score.

### 7.5.2 Consumer Load vs PU Cost Scatter

Scatter plot of consumer demand (X) vs per-unit generation cost (Y). Each point = one time slot. Positive correlation (upper-right cluster) = high-priority DR candidate. Flat or negative = consumer load not aligned with expensive periods.

### 7.5.3 Meter Consumption Heatmap

2D heatmap of selected meter's consumption (Hour of day × Day of year). Compare with the DF savings heatmap to identify overlap between this consumer's peak hours and the highest-value DR windows.

## 7.6 Generator utilisation

This provides generating unit wise Plant Load Factor (PLF), minimum and maximum power delivered.

Heatmap is created for each plant to show percentage contribution of plant generation to total demand.

## 7.7 IEX Market MCP Data Trend Page

Provides market price benchmarking using IEX DAM and RTM data. The page operates in two modes selectable via a radio button:

| Mode | When to Use |
| --- | --- |
| Live Fetch (local only) | Use on a local machine with direct internet access. Scrapes IEX website automatically for the past 7 days. |
| Upload CSV manually | Use on the online (Streamlit Cloud) version, or when live fetch is blocked. Download the CSV from iexindia.com and upload it directly. |

### Why Live Fetch May Fail on the Online Version

Streamlit Cloud's server IP may be blocked by IEX's bot protection or rate limiter, especially after multiple requests in a short period.

### 7.7.1 DAM MCP Trend Chart

Multi-line chart of Day-Ahead Market clearing price (₹/kWh) for each date in the loaded dataset. One coloured line per date. Unified hover mode allows comparison of prices across all dates at a given time block.

### 7.7.2 RTM MCP Trend Chart

Same chart format applied to Real-Time Market prices. RTM prices are more volatile than DAM. Very high RTM spikes indicate grid stress — periods where demand reduction would have maximum value.

# 8. Troubleshooting

## 8.1 Common Errors and Solutions

| Error / Symptom | Likely Cause | Resolution |
| --- | --- | --- |
| LP solver returns 'Infeasible' | Technical minimum exceeds net demand, or flexibility bounds too tight | Check Σ(Technical_min) < min(net_demand). Widen flexibility bounds or check RE profiles |
| Progress bar stuck or very slow | Large num_slots (e.g. 35040) with many plants | Reduce num_slots for testing. Run overnight for full annual runs. |
| FileNotFoundError for output CSV | OUTPUT_PATH in generation_cost2.py does not exist | Edit OUTPUT_PATH to a valid writable path on your system |
| Session state lost after page refresh | Streamlit session state is in-memory only | Re-upload files and re-run analysis. Never refresh during a run. |
| 'KeyError: rate' in market rate loading | market_rate.xlsx column not named 'rate' | Rename the rate column to exactly 'rate' (lowercase) |
| Meter data upload fails on online version | Old code used file path text box instead of file uploader | Use the updated 3_consumer_analysis.py which uses st.file_uploader |
| RE forecast solar is all zeros | Only Wind plants were configured | Add at least one Solar or Solar+Wind plant |
| ANN cost curve not decreasing | Learning rate too high or too low | Try 500 as a starting point. If diverging, reduce by 10×. If flat, increase by 2–5×. |
| ANN test R² is negative | Model is worse than predicting the mean — likely underfitting | Increase iterations, add hidden layers, or check input data normalisation |

## 8.2 Performance Tips

- For initial testing, use a short period (e.g. one week = 168 hourly slots) to verify results before full annual runs.

- Reduce flexibility levels during testing — e.g. [0, 0.10] only — to cut solve time.

- Ensure availability.xlsx has no blank cells. Replace all blanks with 1 (available) or 0 (unavailable).

- For RE forecasting, start with 5 epochs and increase only if test metrics are poor. More epochs = longer training but better fit.

- For ANN forecasting, the default architecture (N_features → N_features → 1) is a good starting point. Add one hidden layer (e.g. 5→8→4→1) if underfitting.

- Use a machine with at least 8 GB RAM for full annual LP runs with more than 5 generating plants.

# 9. Model File Structure

| File | Type | Purpose |
| --- | --- | --- |
| app.py | Python | Main page — file upload, UI parameter config, LP optimization, cost summary |
| 0.RE_forecast.py | Python | RE Forecasting — plant config, LSTM training, solar/wind forecast, RE.xlsx download |
| 1.ANN_forecasting.py | Python | ANN Forecasting — generic time-series forecast, cost curve, test charts, year-ahead forecast |
| 2.charts.py | Python | Flexibility Analysis — generation stack chart, demand heatmap, DF savings heatmap |
| 3.consumer_analysis.py | Python | Consumer Analysis — meter risk scoring, scatter plot, heatmap (uses file uploader) |
| 4.generator_utilisation | Python | Generator PLF, contribution in total demand |
| 5.IEX.py | Python | IEX Market Data — live fetch + CSV upload fallback, DAM/RTM trend charts |
| generation_cost2.py | Python | Core LP optimization engine — builds and solves generation scheduling problem |
| generation_cost.py | Python | Earlier optimizer version (reference only, not used in current app) |
| analyze_high_cost_contributors.py | Python | Consumer meter risk analysis module |
| iex_data.py | Python | IEX market data scraper (used in local mode on IEX page) |
| month_info.py | Python | Calendar utility — returns day ranges for each month in a given year |
| requirements.txt | Text | Python package dependencies |

# 10. Glossary

| Term | Definition |
| --- | --- |
| DISCOM | Distribution Company — the licensed entity responsible for electricity distribution and supply to end consumers in India |
| DSM | Demand-Side Management — programmes aimed at influencing consumer electricity usage patterns |
| DF (Demand Flexibility) | The ability to shift demand across time slots within a day while keeping daily consumption constant |
| DR (Demand Response) | Temporary load curtailment or reduction in response to a grid signal or economic incentive |
| LP (Linear Programming) | A mathematical optimization technique that finds the minimum-cost solution subject to linear constraints |
| PuLP | An open-source Python library for formulating and solving linear programming problems |
| PPA (Power Purchase Agreement) | A contract between a DISCOM and a generator specifying capacity, cost, and supply terms |
| BESS (Battery Energy Storage System) | A grid-scale battery used to store energy during low-cost periods and discharge during high-cost periods |
| IEX | Indian Energy Exchange — India's electricity spot market |
| DAM | Day-Ahead Market — IEX market segment where power for the next day is traded one day in advance |
| RTM | Real-Time Market — IEX market segment where power is traded close to the actual delivery hour |
| MCP | Market Clearing Price — the equilibrium price at which buy and sell bids are matched on the IEX exchange |
| LSTM | Long Short-Term Memory — a type of recurrent neural network well-suited to learning from sequential time-series data |
| ANN | Artificial Neural Network — a multi-layer feedforward network used here for year-ahead time-series forecasting |
| GHI | Global Horizontal Irradiance — solar radiation received per unit area on a horizontal surface (kWh/m²) |
| POA | Plane of Array Irradiance — solar radiation striking the tilted surface of a PV panel |
| Transposition Factor | Ratio of POA to GHI — used to convert horizontal irradiance to panel-incident irradiance |
| Technical Minimum | The lowest generation level at which a thermal or gas plant can operate stably |
| Ramping Rate | The maximum rate at which a generator can increase or decrease output between consecutive time slots |
| Net Demand | Gross consumer demand minus renewable energy generation |
| SoC (State of Charge) | The current energy stored in the battery, in MWh |
| Per-Unit Cost | Total generation cost divided by total energy served (₹/kWh or ₹/MWh) |
| Risk Score | A composite 0–100 score indicating how much a consumer meter contributes to high-cost generation periods |
| Demand Modulation | The difference between base-case and flexibility-adjusted optimised demand at each time slot |
| DF Savings (₹/kWh) | The cost reduction per unit of demand modulation — the value signal for incentivising demand shift |
| nRMSE | Normalised Root Mean Squared Error — RMSE expressed as a percentage of the data range |
| sMAPE | Symmetric Mean Absolute Percentage Error — a scale-independent forecast accuracy metric |
| He Initialisation | A weight initialisation method for ReLU networks that scales weights by √(2/fan_in) to maintain stable gradient flow |

# 11. Support & Maintenance

For technical issues, model configuration queries, or enhancement requests, please contact the model development team. When reporting issues, include:

- The exact error message or symptom observed

- Which page of the application the error occurred on

- The model parameter values used (flexibility levels, battery specs, slots per day)

- The number of time slots and generating plants in the model

- For RE/ANN forecasting issues: number of plants, epochs used, and error at which stage (upload/train/forecast)